VLAN (Virtual Local Area Network)

Contents

What is a VLAN

VLAN Technology

Additional Advantages of Using VLANs

Summary

A VLAN, or Virtual Local Area Network, is a network technology that allows multiple logically separated networks to be created within a single physical network.

The purpose of VLAN technology is to increase network security, improve performance, simplify network management, and make network architecture more flexible.

VLANs allow devices located in different physical locations but belonging to the same department (e.g., HR, Finance, IT) to operate within the same virtual network,

even if they are connected to different switches. Unlike traditional local networks—where segmentation required physical devices such as separate switches—VLAN provides

a software-based solution.

VLAN technology is based on the IEEE 802.1Q standard, which allows network frames to be tagged with a VLAN identifier (VLAN ID).

This VLAN ID is a 12‑bit field, allowing up to 4096 possible VLANs (0–4095, with some values reserved).

There are two main types of VLANs: static (port-based) and dynamic. In static VLANs, the network administrator manually assigns switch ports to a VLAN.

This method is simple and easy to manage but less flexible in large networks. In dynamic VLANs, the system automatically assigns users to VLANs based on MAC address

or username, managed by a server such as the VLAN Management Policy Server (VMPS).

One of the biggest advantages of VLANs is security. Broadcast packets sent by devices in one VLAN do not reach other VLANs.

For example, an attacker cannot see the traffic of another VLAN, helping protect sensitive data. VLANs also improve performance by reducing broadcast domains,

resulting in less network noise and fewer collisions.

Communication between different VLANs does not happen automatically. Inter‑VLAN routing is required, performed by a Layer 3 device such as a router or Layer 3 switch.

Inter‑VLAN routing allows, for example, communication between the HR VLAN and the IT VLAN with proper permissions.

The most commonly used inter‑VLAN routing method is called router‑on‑a‑stick. In this setup, multiple logical sub‑interfaces are created on a single physical router interface,

each with its own VLAN ID. This method is cost‑effective but not ideal for environments with heavy traffic.

Other advantages of VLANs include easier management and scalability. For example, when a new employee joins the finance department,

the administrator simply assigns the proper VLAN ID to the switch port, and the device becomes part of the correct network —

much easier than creating a new physical network.

VLANs are often used alongside technologies such as QoS (Quality of Service) or VoIP, placing voice traffic into a separate VLAN to guarantee bandwidth

and minimize latency.

However, VLANs do not replace firewalls or other security tools. VLANs provide logical separation but do not offer full protection against network attacks.

In summary, VLANs are powerful tools that enable logical network structuring, improved performance, increased security, and easier management —

making them essential in modern enterprise IT infrastructures.

Created by: Kotlár Mátyás Levente